# Analog Communications — Unit 1.3 & 1.4 Revision Notes

### Covers: Reception of AM Signals · Angle Modulation Basics · FM Bandwidth · Indirect FM Generation · Amplitude Modulator Circuits

## PART A — Unit 1.3: Reception of AM Signals

### 1. Methods of Detection

| Type | Description |
| --- | --- |
| Non-Coherent Detection | No need to generate carrier at the receiver for demodulation. Simple, cheap. Used for AM (envelope detection). |
| Coherent Detection | Carrier is locally generated at the receiver for demodulation (needs synchronization with incoming carrier). Used for DSB-SC, SSB. |

AM broadcast uses non-coherent (envelope) detection because the carrier is transmitted along with the sidebands — no need to regenerate it.

### 2. Simple Diode Detector (Envelope Detector)

Circuit: Diode in series → parallel RC (capacitor C and resistor R) → output.

Basic operation example: Assume fc = 300 KHz, fm = 2 KHz

- Frequencies present: 298, 300, 302 KHz (fc − fm, fc, fc + fm)

- The diode is a non-linear device → detects many frequencies: AM frequencies + AM harmonics + SUM of AM frequencies + DIFFERENCE of AM frequencies

- The RC low pass filter (LPF) is set to pass only the DIFFERENCE frequency → this recovers the original modulating signal.

Circuit stages:

- Half-wave Rectifier (the diode D) — removes negative half of the AM wave.

- Smoothing Circuit (C and R in parallel) — smooths the rectified pulses into the envelope = recovered message signal m(t).

This overall circuit is called the Envelope Detector (AM Demodulator/Detector).

Signal flow (waveforms at each stage): DSBAM (m ≤ 1) → Half-wave rectified → RC filter → Smoothed output m(t)

### 3. Envelope vs Detected Envelope

- Envelope: the ideal smooth curve joining the peaks of the AM waveform.

- Detected Envelope: the actual staircase-like output the diode+RC circuit produces, made of small "sawtooth" segments as C charges (through diode) and discharges (through R) between successive RF peaks.

The diode charges C quickly to each peak; between peaks C discharges through R, giving a slight ripple/step approximation of the true envelope.

### 4. Effect of RC Time Constant

The choice of R and C (time constant τ = RC) is critical:

Case 1: RC too LARGE

- C cannot discharge fast enough to follow a falling envelope.

- Output misses peaks — detected envelope lags behind and cuts across envelope troughs ("diagonal clipping").

- Signal at output does not follow the envelope properly (see "Effect of RC being too large" — the output shows a straight diagonal line cutting through a dip instead of following it).

Case 2: RC too SMALL

- C discharges too fast between RF cycles.

- Output still contains significant ripple at carrier frequency — poor smoothing, output looks jagged and closely tracks the carrier ripple rather than a smooth envelope.

Design goal: RC must be chosen so that: $$\frac{1}{f_c} \ll RC \ll \frac{1}{f_m}$$ i.e., large enough to smooth out carrier ripple, but small enough to track the fastest changes in the modulating signal (fm).

### 5. Detailed Diode Detector Waveform Stages

From the labelled diagram (t₀, t₁, t₂, t₃ instants, Vmax, Vmin):

- (a) Input signal vin(t) vs Output signal vout(t) — output follows peaks and decays exponentially between them (sawtooth-on-envelope pattern).

- Shows how the capacitor charges quickly at each positive peak (through low diode forward resistance) and discharges slowly through R until the next peak arrives.

- Final row (d) shows the recovered staircase approximating the modulating signal.

### 6. Concept of Intermediate Frequency (IF) — "Master-Chef Analogy"

The Problem: A radio receiver must handle many different incoming RF signals across a wide band (e.g., 88–108 MHz for FM). Building filters/amplifiers optimized for every possible frequency would be complex and expensive — just like a chef building a custom peeling machine for every different vegetable.

The IF Solution: Convert (pre-process) every incoming signal down to one fixed standard frequency — the Intermediate Frequency (IF) — regardless of which station/channel was tuned. Like the chef's assistant pre-cutting every vegetable into a standard 1-inch cube before it reaches the chef.

The Benefit: Only one set of high-quality, precisely-tuned IF filters/amplifiers is needed, optimized for just that single IF value — much simpler and cheaper than making variable-frequency high-Q filters that track the tuning dial.

In Radio Terms:

- Incoming RF (e.g., 88–108 MHz for FM) → converted down to a fixed IF (10.7 MHz for FM).

- Filters and amplifiers are then optimized for just that one number.

### 7. Standard IF Frequencies (IMPORTANT — memorize for exams)

| Receiver Type | Tuning Range | Standard IF |
| --- | --- | --- |
| AM broadcast | 540 – 1650 KHz | 438–465 KHz range; 455 KHz most popular |
| AM, SSB | — | 1.6 to 2.3 MHz |
| FM broadcast | 88 – 108 MHz | 10.7 MHz |
| TV receivers | — | 26 to 46 MHz |
| Microwave / RADAR | — | 70 MHz |

### 8. Superheterodyne Receiver

Core question: When you tune your radio from 98 MHz to 102 MHz, what actually changes inside?

- The signal frequency (RF) changes as you tune.

- But the internal processing frequency (IF) stays fixed.

- Instead of re-adjusting every filter/amplifier for each station, we convert every incoming signal to one common frequency (IF). That's the "magic" of superheterodyne — hetero (different) + dyne (mixing/beating).

Core formula: $$f_{IF} = |f_{RF} - f_{LO}|$$

- $f_{RF}$ = incoming signal frequency (the station you're tuned to)

- $f_{LO}$ = Local Oscillator frequency (generated inside receiver, tracks with tuning)

- $f_{IF}$ = fixed Intermediate Frequency output

Mixer operation: A mixer is a non-linear device. When $f_{RF}$ and $f_{LO}$ are combined in the mixer, the output contains:

- Sum: $f_{RF} + f_{LO}$

- Difference: $f_{RF} - f_{LO}$

The IF bandpass filter after the mixer is tuned to pass only the difference frequency ($f_{IF} = f_{RF} - f_{LO}$), rejecting the sum and other unwanted mixer products.

### 9. Superheterodyne Receiver — Block Diagram (4 main functional groups)

Antenna → [1. RF Section] → [2. Mixer/Converter Section] → [3. IF Section] → [4. Audio Detector + Amplifier Section] → Speaker

① RF Section:

- Preselector — Broad tuned bandpass filter, adjustable frequency, tuned to carrier frequency.

- Provides initial band limiting to prevent unwanted image frequency from entering the receiver.

- Reduces noise bandwidth; first step in narrowing overall receiver bandwidth to minimum needed.

- RF Amplifier — Determines the sensitivity of the receiver.

- First active device in the chain → primary contributor to noise; predominant factor in the receiver's noise figure.

- A receiver may use one or more RF amplifier stages depending on desired sensitivity.

- Benefits due to RF amplifier:

- Greater gain and better sensitivity

- Improved image frequency rejection

- Better signal-to-noise ratio

- Better selectivity

② Mixer/Converter Section — consists of two components:

- Mixer — where heterodyning (frequency mixing) takes place.

- Radio frequencies are down-converted to intermediate frequency.

- Carrier and sidebands are translated to a new (lower) frequency without affecting the envelope of the message signal (i.e., modulation info is preserved).

- Local Oscillator (LO) — generates $f_{LO}$; tuned together ("ganged tuning") with the RF preselector so that the difference $f_{RF} - f_{LO}$ stays constant (= IF) across the whole tuning range.

③ IF Section:

- Bandpass filter — passes only the IF (difference frequency), rejects sum and image products.

- IF Amplifier(s) ("IF strip") — provides most of the receiver's gain and selectivity since it operates at one fixed frequency, allowing very sharp, well-optimized filtering.

④ Audio Detector + Audio Amplifier Section:

- AM/Audio Detector — recovers the original audio/message signal from the IF signal (envelope detector for AM).

- Audio Amplifier — boosts recovered audio to drive the speaker.

### 10. Frequency Conversion — High-Side vs Low-Side Injection

The difference between RF and LO frequency is always the constant IF.

| Injection Type | LO tuned... | Formula |
| --- | --- | --- |
| High-side injection | Above RF | $f_{LO} = f_{RF} + f_{IF}$ |
| Low-side injection | Below RF | $f_{LO} = f_{RF} - f_{IF}$ |

### 11. Worked Example 1 — RF to IF Conversion (Band-switching AM receiver)

Given: Receiver RF input range 535–1605 KHz split into 3 bands of 10 KHz each (illustrative): Band 1 (535–545), Band 2 (545–555), Band 3 (555–565 KHz).

- Preselector passes 535–565 KHz band.

- Local Oscillator = 1005 KHz (high-side injection, since $f_{LO} > f_{RF}$).

- Mixer output (difference, $f_{IF} = f_{LO} - f_{RF}$) maps the 3 input bands to: 440–450, 450–460, 460–470 KHz.

- IF filter passes 450–460 KHz band → filtered/final IF output centered at 455 KHz (the standard AM IF).

Key formula shown: $f_{LO} = f_{RF} + f_{IF}$ (high-side injection).

### 12. Worked Example 2 — RF to IF Conversion (single-frequency AM channel)

Given:

- $f_c = 900$ KHz (carrier), with $f_{usf} = 905$ KHz (upper sideband), $f_{lsf} = 895$ KHz (lower sideband)

- Preselector passes 895–905 KHz (RF input band)

- Local Oscillator = 1355 KHz, tuning range 995–2055 KHz ("ganged tuning" with preselector), using $f_{LO} = f_{RF} + f_{IF}$

- Difference frequencies at mixer output: 450, 455, 460 KHz

- Bandpass filter (difference) passes 450–460 KHz → standard IF output centred at 455 KHz, sent to IF amplifiers.

$$f_{LO} = f_{RF} + f_{IF} = f_{RF} + 455\ \text{kHz}$$

### 13. IF Calculation Practice Table (fill-in-the-blank style — solve these!)

Using $f_{IF} = |f_{RF} - f_{LO}|$ and $f_{LO} = f_{RF} + f_{IF}$ (assume high-side injection, standard AM IF = 455 KHz unless LO given):

| RF (KHz) | LO (KHz) | IF (KHz) | Notes |
| --- | --- | --- | --- |
| 900 | ? | 455 | LO = 900 + 455 = 1355 |
| 1200 | ? | 455 | LO = 1200 + 455 = 1655 |
| ? | 1805 | 455 | RF = 1805 − 455 = 1350 |
| 1350 | 1705 | ? | IF = 1705 − 1350 = 355? (check sign — verify against std IF; likely low-side or different IF value intended, recompute per given LO) |

Second practice table (all with IF = 455 KHz, find LO):

| RF (KHz) | IF (KHz) | LO (KHz) = RF + IF |
| --- | --- | --- |
| 600 | 455 | 1055 |
| 900 | 455 | 1355 |
| 1100 | 455 | 1555 |
| 1400 | 455 | 1855 |
| 1600 | 455 | 2055 |

(Formula used: $f_{LO} = f_{RF} + f_{IF}$, high-side injection, standard broadcast AM.)

## PART B — Unit 1.4: Angle Modulation — Basic Principles

### 1. What is Angle Modulation?

Frequency Modulation (FM) and Phase Modulation (PM) are both forms of angle modulation.

General equation for an angle-modulated wave: $$m(t) = V_C \cos[\omega_c t + \theta(t)]$$

Where:

- $V_C \cos\theta$ → amplitude term, constant amplitude

- $\omega_c t$ → carrier frequency term

- $\theta(t)$ → the angle term (varies with modulating signal) — this is what makes it "angle" modulation

- $\theta(t) = F[v_m(t)]$ → θ(t) is some function of the modulating signal $v_m(t)$

Angle modulation results whenever the phase angle θ(t) of the signal changes.

### 2. Advantages & Disadvantages of Angle Modulation

Advantages:

- Noise reduction

- Improved system fidelity

- More efficient use of power

Disadvantages:

- Wider bandwidth

- More complex circuits

### 3. FM vs PM — Which is Varied?

- When the frequency of the carrier is varied → FM results.

- When the phase of the carrier is varied → PM results.

- Key relationship: When frequency of the carrier is varied, the phase is also varied, and vice versa — the two are interlinked.

Direct/Indirect relationship (important conceptual point):

Direct FM is indirect PM, and Direct PM is indirect FM.

- Direct FM: Varying the frequency of a constant-amplitude carrier directly proportional to the amplitude of the modulating signal.

- Direct PM (= Indirect FM): Varying the phase of a constant-amplitude carrier directly proportional to the amplitude of the modulating signal.

### 4. Angle Modulated Wave — Frequency Deviation Concept

Given carrier at $f_c$ = 100 KHz, with modulating signal $v_m(t) = V_m \sin(2\pi f_m t)$:

- When $v_m(t) = +V_m$ (positive peak) → carrier frequency shifts to $f_c + \Delta f$ = 105 KHz

- When $v_m(t) = -V_m$ (negative peak) → carrier frequency shifts to $f_c - \Delta f$ = 95 KHz

- $\Delta f$ = frequency deviation = 5 KHz in this example

This is the essence of FM: the instantaneous carrier frequency swings above and below $f_c$ in proportion to the modulating signal amplitude.

### 5. Visualizing Phase vs Frequency Changing with Time

Phase changing with time: As the modulating signal $v_m(t)$ varies, the carrier's phase is shifted by $\pm\Delta\theta$ relative to the unmodulated carrier — visible as the carrier waveform shifting left/right (in time) relative to a reference sinusoid.

Frequency changing with time:

- Low amplitude of input signal → low frequency change in carrier

- High amplitude of input signal → high frequency change in carrier

- Unmodulated carrier: constant amplitude, constant frequency, constant phase.

- FM wave (modulated): constant amplitude, "constant phase" (in the sense of continuous phase — no discontinuities), but frequency changes according to amplitude of the input signal.

- Period $T_{max}$ occurs where the input signal amplitude is such that instantaneous frequency is lowest (wave "stretched out" — longer period); $T_{min}$ occurs where instantaneous frequency is highest (wave "compressed" — shorter period).

### 6. Derivation of the FM Wave Equation (IMPORTANT — derivation asked in exams)

Step 1: Instantaneous frequency is given by: $$\omega_i = \omega_c + \Delta f$$

Since $\Delta f \propto v_m(t)$: $$\Delta f = K_f v_m(t) \quad \text{...(1)}$$

Where $K_f$ = deviation sensitivity of FM wave (units: rad/sec per volt): $$K_f = \frac{\Delta \omega}{\Delta V} \ \left(\frac{\text{rad/sec}}{\text{volts}}\right)$$

Step 2: Let $v_m(t) = V_m \cos\omega_m t$. Substituting into (1): $$\Delta f = K_f V_m \cos\omega_m t$$ $$\omega_i = \omega_c + K_f V_m \cos\omega_m t \quad \text{...(2)}$$

Step 3: The general FM wave is $v_{fm}(t) = V_c \cos\theta$ ...(3), where θ is the total instantaneous angle (frequency + phase combined).

Since instantaneous frequency $\omega_i = \dfrac{d\theta}{dt}$, we get $\theta = \int \omega_i , dt$.

Step 4: Integrate: $$\theta = \int (\omega_c + K_f V_m \cos\omega_m t), dt = \omega_c t + \frac{K_f V_m}{\omega_m}\sin\omega_m t$$

Since $\Delta f = K_f V_m$ (peak frequency deviation): $$\theta = \omega_c t + \frac{\Delta f}{\omega_m}\sin\omega_m t$$

Define modulation index $m_f = \dfrac{\Delta f}{\omega_m}$ (technically $\Delta f / f_m$, both are Hz so it's unitless): $$\boxed{\theta = \omega_c t + m_f \sin\omega_m t}$$

Step 5: Final FM Wave Equation: $$\boxed{v_{fm}(t) = V_c \cos[\omega_c t + m_f \sin\omega_m t]}$$

This is the fundamental equation used to expand FM waves using Bessel function identities (see Frequency Spectrum section below).

### 7. AM vs FM — Modulation Index Comparison

|  | AM | FM |
| --- | --- | --- |
| Modulation index | $m = \dfrac{V_m}{V_c}$ | $m_f = \dfrac{\Delta f}{f_m}$ |
| Nature | Ratio of voltages | Ratio of frequencies |
| What varies | Voltage changes | Frequency changes |

This is a key conceptual distinction for exams: AM modulation index compares amplitudes, while FM modulation index compares the frequency deviation to the modulating frequency.

### 8. Instantaneous Phase, Frequency & Deviation — Four Key Definitions

① Instantaneous Phase The precise phase of the carrier at a given instant of time: $$\text{Instantaneous phase} = \omega_c t + \theta(t) \ \ \text{rad}$$

② Instantaneous Phase Deviation The instantaneous change in the phase of the carrier at a given instant, with reference to the unmodulated phase: $$\text{Instantaneous phase deviation} = \theta(t) \ \ \text{rad}$$

③ Instantaneous Frequency The precise frequency of the carrier at a given instant — first time derivative of instantaneous phase: $$\omega_i(t) = \frac{d}{dt}[\omega_c t + \theta(t)] = \omega_c + \theta'(t)\ \ \text{rad/s}$$

④ Instantaneous Frequency Deviation The instantaneous change in frequency — first time derivative of the phase deviation: $$\text{Instantaneous frequency deviation} = \theta'(t)\ \ \text{rad/s}$$

Converting to Hz: $$\frac{\theta'(t)\ \text{rad/sec}}{2\pi\ \text{rad/cycle}} = \frac{\text{cycles}}{s} = \text{Hz}$$

### 9. Deviation Sensitivity — PM vs FM (VERY IMPORTANT for exams)

Phase Modulation (PM):

- Instantaneous phase deviation is proportional to the amplitude of the modulating signal voltage.

- Instantaneous frequency deviation is proportional to the slope (first derivative) of the modulating signal. $$\theta(t) = K_p v_m(t)\ \ \text{rad}$$

Frequency Modulation (FM):

- Instantaneous frequency deviation is proportional to the amplitude of the modulating signal voltage.

- Instantaneous phase deviation is proportional to the integral of the modulating signal voltage. $$\theta'(t) = K_f v_m(t)\ \ \text{rad}$$

$K_p$ and $K_f$ are called deviation sensitivities — they represent the input-versus-output transfer function of the modulator.

Formal deviation sensitivity definitions:

For a phase modulator: $$K = \frac{\text{rad}}{V}\left(\frac{\Delta\theta}{\Delta V}\right)$$

For a frequency modulator: $$K_1 = \frac{\text{rad/s}}{V}\left(\frac{\Delta\omega}{\Delta V}\right)$$

### 10. Phase Deviation and Modulation Index

For a carrier phase- or frequency-modulated by a single-frequency modulating signal: $$m(t) = V_c \cos[\omega_c t + m\cos(\omega_m t)]$$

Here m represents peak phase deviation in radians for a phase-modulated carrier — called the modulation index: $$m = K_p V_m \ \ (\text{radians})$$ $$m = K_p \left(\frac{\text{radians}}{\text{volt}}\right) V_m (\text{volts}) = \text{radians}$$

### 11. Frequency Deviation — Formal Definitions

Frequency Deviation: The change in frequency that occurs in the carrier when acted on by a modulating signal frequency.

- Peak frequency deviation = $\Delta f$

- Peak-to-peak frequency deviation = $2\Delta f$

$$\Delta f = K_f V_m \ \ (\text{Hz})$$

$$m = \left(\frac{\Delta f (\text{Hz})}{f_m (\text{Hz})}\right) \ \ (\text{unitless})$$

## PART C — Unit 1.4: FM Frequency Spectrum & Bandwidth Calculations

### 1. Frequency Spectrum of FM Wave (Bessel Expansion)

Starting from: $$v_{fm}(t) = V_c \cos[\omega_c t + m_f \sin\omega_m t]$$

Using Bessel function identities, this expands into an infinite series of sidebands:

$$v_{fm}(t) = V_c J_0(m_f)\cos\omega_c t$$ $$+ J_1(m_f){\cos(\omega_c+\omega_m)t - \cos(\omega_c-\omega_m)t}$$ $$+ J_2(m_f){\cos(\omega_c+2\omega_m)t - \cos(\omega_c-2\omega_m)t}$$ $$+ J_3(m_f){\cos(\omega_c+3\omega_m)t - \cos(\omega_c-3\omega_m)t}$$ $$+ \ldots + J_n(m_f)$$

Where $J_n(m_f)$ = Bessel function of the first kind, order n, argument $m_f$.

Interpretation:

- $J_0(m_f)$ → amplitude of the carrier component

- $J_1(m_f), J_2(m_f), \ldots$ → amplitudes of the 1st, 2nd, 3rd... pairs of sidebands at $f_c \pm nf_m$

### 2. Frequency Spectrum Diagram (Ideal FM Spectrum)

Spectrum shows lines at: $f_c - 5f_m, f_c-4f_m, \ldots, f_c-f_m, f_c, f_c+f_m, \ldots, f_c+5f_m$

- Lower sidebands to the left of $f_c$, Upper sidebands to the right.

- Amplitude of carrier at $f_c$ = $J_0(m_f)$

- Amplitude of 1st sideband pair = $J_1(m_f)$ (both sides), 2nd = $J_2(m_f)$, etc.

- Theoretically, FM has an infinite number of sidebands → for an ideal FM spectrum, Bandwidth = ∞.

- In practice, sideband amplitudes decrease and become negligible beyond a certain order — this bounds the practical bandwidth.

### 3. Bessel Function Table (memorize the structure, use in numericals)

| Modulation index $m_f$ | $J_0$ (Carrier) | $J_1$ | $J_2$ | $J_3$ | $J_4$ | $J_5$ | ... |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 1.00 | – | – | – | – | – |  |
| 0.25 | 0.98 | 0.12 |  |  |  |  |  |
| 0.5 | 0.94 | 0.24 | 0.03 |  |  |  |  |
| 1.0 | 0.77 | 0.44 | 0.11 | 0.02 |  |  |  |
| 1.5 | 0.51 | 0.56 | 0.23 | 0.06 | 0.01 |  |  |
| 2.0 | 0.22 | 0.58 | 0.35 | 0.13 | 0.03 |  |  |
| 2.41 | 0 | 0.52 | 0.43 | 0.20 | 0.06 | 0.02 |  |
| 5.53 | 0 | −0.34 | −0.13 | 0.25 | 0.40 | 0.32 | ... |
| 8.65 | 0 | 0.27 | 0.06 | −0.24 | −0.23 | 0.03 | ... |

Key formula relating modulation index to sideband count: $$m_f = \frac{\Delta f}{f_m}$$

How to use the table:

- Look up the row for your calculated $m_f$.

- Each column gives you the relative amplitude of that sideband order.

- Significant sidebands are conventionally those with amplitude ≥ 0.01 (1%) of unmodulated carrier — used to determine practical bandwidth (n = number of significant sideband pairs).

- Note: $J_0$ can go negative (e.g., at $m_f = 2.41$, $J_0 = 0$ — carrier completely disappears! This is used practically to measure deviation in FM generators).

### 4. Frequency Spectrum vs Modulation Index (n = number of sideband pairs)

| $m_f$ | n (number of significant sidebands) |
| --- | --- |
| 0.5 | 1 |
| 1.0 | 2 |
| 2.0 | 4 |
| 4.0 | 5 |

As $m_f$ increases, more sideband pairs become significant, and the spectrum spreads wider.

### 5. Bessel Function Curves ($J_n(m)$ vs modulation index m)

- Plot shows $J_0(m), J_1(m), \ldots J_8(m)$ as oscillating, decaying curves vs modulation index m (0 to 12).

- All curves start together: $J_0(0) = 1$, all higher orders = 0 at $m=0$ (unmodulated case — only carrier present).

- As $m$ increases, energy spreads from $J_0$ into higher-order $J_n$ terms — physically meaning more sidebands carry significant power as modulation index increases.

- Each $J_n(m)$ curve oscillates and crosses zero at various points, and generally decays in peak amplitude as order n increases (for fixed range of m shown).

### 6. Bandwidth of FM Wave

Bandwidth of FM wave is a function of modulating signal and modulation index.

Case 1: Low modulation index (m < 1) → behaves similar to AM: $$\text{Bandwidth} = 2f_m \ \ \text{Hz}$$

Case 2: High modulation index (m > 1): $$\text{Bandwidth} = 2\Delta f \ \ \text{Hz}$$

### 7. Actual Bandwidth (using significant sidebands) & Carson's Rule

Actual bandwidth required to pass all significant sidebands = two times the product of (number of significant sidebands) × (highest modulating signal frequency): $$\text{Bandwidth} = 2(n \times f_m) \ \ \text{Hz}$$

Carson's Rule — a widely used approximation for FM bandwidth: $$\boxed{\text{Bandwidth} = 2(\Delta f + f_m) \ \ \text{Hz}}$$

This is the most commonly used formula in numerical problems (balances practicality vs the infinite theoretical bandwidth).

### 8. FM is Called a "Constant Bandwidth System" — Worked Cases

Using Carson's Rule $BW = 2(\Delta f + f_m)$, with fixed $\Delta f$ = 75 KHz (standard commercial FM peak deviation):

| Case | $\Delta f$ | $f_m$ | Bandwidth = $2(\Delta f + f_m)$ |
| --- | --- | --- | --- |
| 1 | 75 KHz | 500 Hz | $2(75000+500) = 151{,}000$ Hz = 151 KHz |
| 2 | 75 KHz | 5000 Hz | $2(75000+5000) = 160{,}000$ Hz = 160 KHz |
| 3 | 75 KHz | 10000 Hz | $2(75000+10000) = 170{,}000$ Hz = 170 KHz |

Key insight: Even though $f_m$ varies significantly (500 Hz → 10 KHz, a 20× change), the total bandwidth stays roughly close to the standard 200 KHz FM channel allocation, since $\Delta f$ (75 KHz, set by the deviation limit) dominates the sum. This is why FM broadcast is called a "constant bandwidth system" — the channel bandwidth is primarily set by the fixed peak deviation, not strongly by the audio frequency being transmitted (within audio range).

### 9. Advantages of FM

- Less noise and interference

- Compared to AM, FM includes low power consumption

- The radiated power is less

- Guard bands separate nearby FM channels

- Lesser geographical interference among adjacent stations

### 10. Disadvantages of FM

- High bandwidth

- Receiving area of the FM signal is small

- The antennas for FM systems should be kept close for better communication

- More complicated receiver and transmitter

- Poor spectral efficiency

## PART D — Indirect FM Generation (Armstrong Method)

### 1. Overview: Armstrong Method — Divided into 3 Parts

- Part 1: Generate NBFM (Narrow Band FM) signal using a phase modulator.

- Part 2: Implementation of the Phase Modulator.

- Part 3: Use of frequency multipliers, mixers, and amplifiers to convert NBFM → WBFM (Wide Band FM).

Basic block diagram (simplified):

Modulating Signal → [NBFM Modulator] → NBFM wave → [Frequency Multiplier] → WBFM wave → Transmit

↑

Carrier: Ac cos(2π fc t)



### 2. Full Basic Block Diagram (detailed)

Crystal Oscillator (Carrier) → phase modulator (PM) → frequency multiplier → Power amplifier → Antenna

↑

Audio source (Modulating Signal)

- Crystal Oscillator → generates stable carrier.

- Phase Modulator → produces NBFM (phase-modulated) output; combines carrier + audio.

- Frequency Multiplier → multiplies frequency to increase deviation, converting NBFM to WBFM.

- Power Amplifier → boosts WBFM signal before transmission via antenna.

### 3. Indirect FM Generation — Full Detailed Block Diagram

Crystal Oscillator → Buffer ─┬──────────────→ Combining Network (Σ) → NBFM/FM wave

│                        ↑

90° Phase Shifter → Balanced Modulator ← Audio Equalizer (Integrator) ← Modulating signal

(only sidebands, 90° with carrier)

NBFM wave → [Group of Multipliers] → [Mixer] → [Group of Multipliers] → [Class C Power Amp] → Transmitting Antenna

↑

Crystal Oscillator (via Frequency Translator)

Key blocks and their roles (Part-3 = right-hand side, labelled "Multipliers and Amplifiers"):

- Buffer — isolates the crystal oscillator from loading effects; passes carrier to both the combining network and the 90° phase shifter.

- 90° Phase Shifter — shifts the carrier by 90° before it enters the balanced modulator.

- Audio Equalizer (Integrator) — integrates the modulating signal before applying it to the balanced modulator (this integration is what makes a phase modulator behave equivalently for indirect FM — recall FM needs phase deviation proportional to the integral of $v_m(t)$).

- Balanced Modulator — combines the 90°-shifted carrier with the equalized (integrated) modulating signal to produce a double-sideband suppressed-carrier (DSB-SC) signal — i.e., only sidebands (USB and LSB), no carrier, at 90° relative phase to the main carrier.

- Combining Network (Σ = Adder) — adds the original (unshifted) carrier to the sidebands-only output of the balanced modulator → produces the phase-modulated (NBFM) wave.

- Group of Multipliers (Part 3, first set) — multiplies frequency (and hence multiplies the phase/frequency deviation) of the NBFM signal, converting NBFM to a larger deviation.

- Mixer — combines with a second crystal oscillator (acting as a frequency translator) to shift the signal to the correct final carrier frequency without changing deviation.

- Group of Multipliers (second set) — further multiplies frequency/deviation as needed to reach the required $\Delta f$ for WBFM.

- Class C Power Amplifier — final high-efficiency amplification stage before the transmitting antenna, output = WBFM.

### 4. Phase Modulator — Detailed Block Diagram & Signal Labels

Crystal Oscillator (A) → Buffer (B) → Carrier (C) ───────────────┐

→ Adder (Σ) → (F) PM wave

↓                                       ↑

90° phase shifter (B→D) → 90° shifted carrier (D) → Product Modulator (E) ← Modulating signal (Audio Source)

(only sidebands at 90° with carrier)

Point-by-point signal labels:

- A = Unmodulated carrier (at crystal oscillator output)

- B = Unmodulated carrier (at buffer output)

- C = Unmodulated carrier (fed to adder)

- D = 90°-shifted carrier (from 90° phase shifter)

- E = Output of Product Modulator / Balanced Modulator = Double Sideband signal — only sidebands (USB, LSB) at 90° with carrier, carrier suppressed

- F = Output of Adder (Σ) = PM wave (final phase-modulated output)

The Product Modulator is also called a Balanced Modulator — it multiplies the 90°-shifted carrier with the modulating (audio) signal to produce a DSB signal with suppressed carrier.

### 5. Phasor Diagram — Generation of PM (VERY IMPORTANT conceptually)

At points A, B, C: Unmodulated carrier — shown as a single vertical phasor (reference).

At D: 90° phase shifter output — the 90°-shifted carrier phasor, rotated 90° from the reference, pointing horizontally (leftward in the diagram).

At E (Balanced modulator output):

- Shows USB and LSB phasors rotating in opposite directions relative to the (now suppressed) carrier position.

- Resultant of sidebands = vector sum of USB + LSB phasors — this resultant always stays perpendicular (in quadrature) to the original carrier direction as it grows/shrinks with modulation, because USB and LSB rotate oppositely and their carrier-direction components cancel while their quadrature components add.

At F (Combining network / Adder output):

- The unmodulated carrier phasor is added vectorially to the resultant of sidebands phasor (from E).

- This vector addition produces the resultant at the output of combining network, which has:

- A slightly different magnitude than the pure carrier (small AM component — this is actually an unwanted side-effect/approximation of the Armstrong method, kept small by keeping $m_f$ small = "narrowband").

- A phase angle φ relative to the original carrier — this φ is the phase deviation produced, which varies with the modulating signal.

- φ = phase angle is shown explicitly in the diagram as the angle between the unmodulated carrier phasor and the resultant phasor at F.

Effect of Vm on phase angle:

- High Vm (large modulating signal amplitude) → larger sideband phasors (USB/LSB) → larger resultant → larger φ (more phase deviation).

- Low Vm → smaller sidebands → smaller resultant → smaller φ.

### 6. Phasor Diagram — Effect of Modulation Index on Frequency

- Low modulation index: small φ (phase deviation) — sidebands phasors are short, resultant phasor stays close to the carrier direction, so φ is small.

- High modulation index: φ increases — sidebands phasors are longer, resultant phasor swings further from carrier direction, so φ is larger.

Chain of causation (as marked in diagram): $$\Delta \phi \leftarrow m_f \rightarrow \Delta f \rightarrow v_m(t)$$

i.e., a change in the modulating signal amplitude $v_m(t)$ → causes a change in frequency deviation $\Delta f$ → which relates to modulation index $m_f$ → which determines the phase change φ produced by the modulator.

This links directly back to Part B's derivation: $m_f = \Delta f / f_m$, and phase deviation is directly tied to $m_f$ in this narrowband PM-based generation scheme.

## PART E — Amplitude Modulator Circuits

### 1. Two Approaches to AM Generation

- Flywheel effect of Tank circuit (L and C) which has a high Q factor.

- Applying modulating signal and carrier signal to a mixer (a non-linear device).

### 2. Approach 1: Non-linear Mixing Approach

Block diagram:

Antenna → LNA → RF → [Down-conversion Mixer] → IF

↑

Local Oscillator (LO)

- Non-Linear Analog device acting as Frequency Translator.

- Modulating Signal (Fm) applied at RF input (after LNA)

- Carrier signal (Fc) applied via Local Oscillator

- Mixer output contains Fc + Fm (sum) and Fc − Fm (difference)

Mathematical basis: If the inputs are sinusoids, the ideal mixer output is the sum and difference frequencies: $$V_o = [A_1\cos(\omega_1 t)][A_2\cos(\omega_2 t)] = \frac{A_1 A_2}{2}[\cos(\omega_1-\omega_2)t + \cos(\omega_1+\omega_2)t]$$

This is the fundamental trigonometric identity behind all mixer/modulator operation — multiplying two sinusoids in a non-linear device produces sum and difference frequency components.

### 3. Approach 2: Flywheel Effect of Tank Circuit

Setup: A tank circuit (parallel LC) is monitored on a CRO (oscilloscope). Even though the input might be a series of current pulses (not a continuous sinusoid), the high-Q tank circuit "rings" and produces a smooth sinusoidal output at its resonant frequency — this is the flywheel effect (analogous to a mechanical flywheel storing rotational energy and smoothing out pulsed input torque into continuous rotation).

Tank circuit charge/discharge cycle (from the 5-stage diagram a–e):

- (a) Capacitor charged (+ on top plate) → current begins flowing out into inductor

- (b) Current flows through inductor, building magnetic field (energy transferring from C to L)

- (c) Capacitor now charged with opposite polarity (− on top, + on bottom) — energy has fully transferred and capacitor recharges in reverse

- (d) Current now flows in the reverse direction through inductor (magnetic field collapsing, driving current back)

- (e) Capacitor returns to original polarity — one complete oscillation cycle

This oscillatory exchange of energy between L and C (with only small losses per cycle, thanks to high Q) is what sustains a nearly-sinusoidal "ringing" waveform even from a pulsed/non-continuous input.

### 4. Flywheel Effect — Input vs Output Waveforms

- (a) Input to tank: A series of current pulses of varying amplitude (pulses 1–12 shown, e.g., increasing then decreasing amplitude envelope — like an AM-modulated pulse train).

- (b) Output of tank (Tuned-circuit AM voltage): The tank circuit's flywheel/ringing effect converts these discrete pulses into a continuous, smooth AM waveform — the envelope of the output sinusoid follows the envelope of the input pulse amplitudes.

Key takeaway: A high-Q tank circuit, when excited by periodic pulses of varying amplitude, generates a continuous AM output whose envelope tracks the pulse amplitude envelope. This is the basis of the "Flywheel effect" approach to AM generation (as opposed to non-linear mixing).

### 5. Medium Power AM Modulator — Basic Circuit (Collector Modulation)

Circuit description:

- Transistor $Q_1$ used as a switch/non-linear amplifying device.

- Unmodulated carrier ($V_c$) fed into the base via transformer $T_2$ (1:1), with $R_1$ and $C_1$ providing biasing/coupling.

- Single-frequency modulating signal ($v_m$, with peak $V_p = V_{CC}$) fed into the collector supply via transformer $T_1$ (1:1) and an RFC (RF Choke).

- Output $V_{out}$ taken from the collector.

- This is called Collector Modulation because the modulating signal is applied at the collector circuit.

Operating principle:

- $V_C > 0.7\text{V} \Rightarrow$ transistor turns ON (base-emitter forward biased), acting as an open/non-linear mixing device when combined with the varying collector supply.

- $V_{CE(sat)} \approx 0.3\text{V}$ — the saturation voltage when transistor is fully ON.

### 6. Waveforms — NO Modulating Signal (baseline case)

- $V_c$ (base carrier drive): sinusoidal, swinging around 0V up to ~0.7V threshold.

- $I_c$ (collector current): flows only during the portion of each carrier cycle when $V_c > 0.7V$ — appears as a series of narrow current pulses (Class-C-like operation).

- $V_{out}$: square-wave-like pulses swinging between $V_{CC}$ (transistor OFF) and $V_{CE(sat)} \approx 0V$ (transistor fully ON/saturated) — with no envelope variation since there's no modulating signal (constant amplitude pulses).

### 7. Waveforms — WITH Modulating Signal (collector modulation)

- $v_m$ (modulating signal, $V_p = V_{CC}$): slow sinusoidal envelope.

- $V_c$ (carrier): fast sinusoidal, same as before — swings around 0.7V threshold.

- $I_c$ (collector current pulses): amplitude of the current pulses now varies following the modulating signal envelope — larger pulses where $v_m$ is near its peak, smaller/no pulses where $v_m$ is near its trough.

- $V_{out}$: Output waveform amplitude now varies between $2V_{CC}$ (Vmax, when both carrier ON and modulating signal at peak) and down toward $V_{CE(sat)} \approx 0V$ (Vmin) — the envelope of $V_{out}$ traces the shape of the AM-modulated wave, confirming successful collector amplitude modulation.

### 8. Medium Power AM Modulator — More Practical Circuit (with Tank/Tuned output)

Additional/refined circuit elements over the basic version:

- Modulating signal now applied through $T_1$ (1:1) with $C_3$, $C_4$ providing RF bypass/coupling at the collector supply side — this stage is called Collector Modulator.

- Bandlimiting RC Circuit ($C_1$, $R_1$, $C_2$) — used to shape/limit the bandwidth of the modulating signal path (base bias network via $T_3$).

- Tank/Tuned circuit ($L_1$ with $C_N$ — a variable/trimmer capacitor) at the collector, coupled via transformer $T_2$ to the Antenna — this tank circuit selects/filters the desired output frequency and smooths the pulsed collector current into a clean sinusoidal AM output for radiation (this is the Flywheel effect in action, applied to the practical modulator circuit).

- Unmodulated carrier ($v_c$) still applied at the base via $T_3$.

This combines both AM generation approaches from Section 1: the transistor+carrier stage acts as the non-linear mixing device, while the output tank circuit ($L_1$, $C_N$) provides the flywheel smoothing effect to produce a clean AM waveform at the antenna.

### 9. Medium Power AM Modulator — Full Waveforms (with Tank Circuit output)

- $V_m$: slow modulating envelope (0 to $V_{CC}$).

- $V_c$: fast carrier sinusoid (swinging 0 to 0.7V threshold region).

- $I_c$: collector current pulses, amplitude-varying with $V_m$ (same pattern as basic circuit — larger pulses at $v_m$ peak).

- $V_{out}$: With the tank circuit now smoothing the pulses (flywheel effect), the output is a genuinely smooth, continuous AM waveform — clean upper envelope at Vmax = +2Vcc and lower envelope at Vmin = −2Vcc, properly following the sinusoidal shape of the modulating signal (unlike the raw pulsed output of the basic circuit). This confirms the tank/tuned circuit's role in converting the switched/pulsed collector waveform into a clean, transmittable AM signal.

## Quick-Reference Formula Sheet

| Concept | Formula |
| --- | --- |
| General angle-modulated wave | $m(t) = V_C\cos[\omega_c t + \theta(t)]$ |
| FM wave (final form) | $v_{fm}(t) = V_c\cos[\omega_c t + m_f\sin\omega_m t]$ |
| Modulation index (FM) | $m_f = \Delta f / f_m$ |
| Modulation index (AM) | $m = V_m/V_c$ |
| Frequency deviation | $\Delta f = K_f V_m$ (Hz) |
| Phase deviation / mod. index (PM) | $m = K_p V_m$ (rad) |
| Instantaneous frequency | $\omega_i = \omega_c + \theta'(t)$ |
| Instantaneous phase deviation | $\theta(t)$ |
| Instantaneous freq. deviation | $\theta'(t)/2\pi$ = Hz |
| BW (low index, m<1) | $2f_m$ |
| BW (high index, m>1) | $2\Delta f$ |
| BW (actual, using sidebands) | $2(n \times f_m)$ |
| Carson's Rule (BW) | $2(\Delta f + f_m)$ |
| Superheterodyne IF | $f_{IF} = \lvert f_{RF}-f_{LO}\rvert$ |
| High-side injection | $f_{LO} = f_{RF}+f_{IF}$ |
| Low-side injection | $f_{LO} = f_{RF}-f_{IF}$ |
| Mixer product (trig identity) | $A_1\cos\omega_1t \cdot A_2\cos\omega_2t = \frac{A_1A_2}{2}[\cos(\omega_1-\omega_2)t+\cos(\omega_1+\omega_2)t]$ |

Standard IF values to memorize: AM broadcast = 455 KHz · AM/SSB = 1.6–2.3 MHz · FM broadcast = 10.7 MHz · TV = 26–46 MHz · Microwave/RADAR = 70 MHz

Compiled from lecture slides: Unit 1.3 (Reception of AM Signals), Unit 1.4 (Angle Modulation, FM Bandwidth Calculations, Armstrong Method of FM Generation), and Amplitude Modulator Circuits — Kartik R. Patel, EXTC Dept, KJSSE/SVU.